Архитектура рачунара,  јул 2011.

Решење задатка

II

а)

| Шта? | Почетна адреса [h] | Последња адреса [h] |
| --- | --- | --- |
| Меморијски адресни простор | 0000 0000 | FFFF FFFF |
| Улазно-излазни адресни простор | 0000 0000 | FFFF FFFF |
| Адресни простор резервисан за RАМ | 0002 0000 | FFFF FFFF |
| Адресни простор резервисан за RОМ | 0000 0000 | 0001 FFFF |
| Физичка RАМ меморија | 4000 0000 | BFFF FFFF |
| Физичка RОМ меморија | 0000 0000 | 0007 FFFF |

б)

в)

IV

Варијанта 1

;	MOV mem/reg, mem/reg/imm ; MOV dst, src

;	OC reg, mem/reg/imm ; OC dst/src1, src2

;	JOC abs

; initialize input from PER0 (fill BP0)

MOV memSem0, #0

MOV memAdr0, #1000h

MOV memCnt0, #100h

MOV FF00h, #A2h

; ...

Wait1:	TST memSem0, #1h

JZ Wait1

; initialize output from BP0 and input to BP1

Swap:	MOV Aout, #1000h

MOV memAdr0, #1100h

MOV Dir, #1h

Init:	MOV memSem0, #0h

MOV memSem1, #0h

MOV memSem2, #0h

MOV memCnt0, #100h

; init DMAs

MOV FF31h, Aout

MOV FF41h, Aout

MOV FF32h, #100h

MOV FF42h, #100h

; start controllers

MOV FF00h, #A2h

MOV FF10h, #2h

MOV FF20h, #2h

MOV FF30h, #82h

MOV FF40h, #182h

;		....

Wait2:	MOV R0, memSem0

AND R0, memSem1

AND R0, memSem2

JZ Wait2

TST Dir, #1

JZ Swap

; initialize output from BP1 and input to BP0

MOV Aout, #1100h

MOV memAdr0, #1000h

MOV Dir, #0h

JMP Init

DMA1Int:

MOV FF10h, #0h

MOV FF30h, #0h

MOV memSem1, #1h

RTI

DMA2Int:

MOV FF20h, #0h

MOV FF40h, #0h

MOV memSem2, #1h

RTI

Per0Int:

MOV (memAdr0), FF02h

INC memAdr0

DEC memCnt0

JNZ Back0

MOV FF00h, #0h	;stop PER0

MOV memSem0, #1h

Back0:	RTI

Варијанта 2

;	LOAD reg, mem/reg/imm ;LOAD dst, src

;	STORE reg, mem/reg ;	STORE src, dst

;	OC reg, mem/reg/imm ; OC dst/src1, src2

;	JOC abs

; initialize input from PER0 (fill BP0)

LOAD R0, #0

LAOD R1, #1000h

LAOD R2, #100h

LOAD R3, #A2h

LOAD R6, #1h

LAOD R7, #1100h

STORE R0, memSem0

STORE R1, memAdr0

STORE R2, memCnt0

STORE R3, FF00h

Wait1:	TST R6, memSem0

JZ Wait1

; initialize output from BP0 and input to BP1

Swap:	STORE R1, Aout

STORE R7, memAdr0

STORE R6, Dir

Init:	STORE R0, memSem0

STORE R0, memSem2

STORE R0, memSem1

STORE R2, memCnt0

; init DMAs

LOAD R9, Aout

STORE R9, FF31h

STORE R9, Aout

STORE R2, FF32h

STORE R2, FF42h

; start controllers

LOAD R9, #A2h

STORE R9, FF00h

LOAD R9, #2h

STORE R9, FF10h

LOAD R9, #2h

STORE R9, FF20h

LOAD R9, #82h

STORE R9, FF30h

LOAD R9, #182h

STORE R9, FF40h

;		....

Wait2:	LOAR R9, memSem0

AND R9, memSem1

AND R9, memSem2

JZ Wait2

LOAD R9, Dir

AND R9, #1

JZ Swap

; initialize output from BP1 and input to BP0

STORE R7, Aout

STORE R1, memAdr0

STORE R0, Dir

JMP Init

DMA1Int:

STORE R0, FF10h

STORE R0, FF30h

STORE R6, memSem1

RTI

DMA2Int:

STORE R0, FF20h

STORE R0, FF40h

STORE R6, memSem2

RTI

Per0Int:

LOAD R9, FF02h

STORE R9, (memAdr0)

LOAD R9, memAdr0

INC R9

STORE R9, memAdr0

LOAD R9, memCnt0

DEC R9

STORE R9, memCnt0

JNZ Back0

STORE R0, FF00h ;stop PER0

STORE R6, memSem0

Back0:	RTI