Архитектура рачунара,  јун 2011.

Решење задатка

II

а)

| Шта? | Почетна адреса [h] | Последња адреса [h] |
| --- | --- | --- |
| Адресни простор | 000000000 | FFFFFFFFF |
| Меморијски адресни простор | 000000000 | EFFFFFFFF |
| Улазно-излазни адресни простор | F00000000 | FFFFFFFFF |
| Адресни простор резервисан за RАМ | 000000000 | EFFFDFFFF |
| Адресни простор резервисан за RОМ | EFFFE0000 | EFFFFFFFF |
| Физичка RАМ меморија | 000000000 | 0FFFFFFFF |
| Физичка RОМ меморија | EFFFE0000 | EFFFE7FFF |

б)

в)

IV

LOAD #0

STORE Cnt

LOAD #100h

STORE Size

LOAD #1000h

STORE Start

STORE Head

STORE Tail

LOAD #1100h

STORE End

INTD

LOAD #20h

OUT F010h

LOAD #20h

OUT F020h

LOAD #A8h

OUT F030h

LOOP0:		LOAD Cnt

CMP Size

JZ LOOP0

PER0:		IN F011h

TST #100h

JZ PER1

IN F012h

STORE (Head)

INC Head

LOAD Head

CMP End

JNZ SKIP0

LOAD Start

STORE Head

SKIP0:		INC Cnt

INTE

LOOP1:		LOAD Cnt

CMP Size

JZ LOOP1

PER1:		IN F021h

TST #100h

JZ PER0

IN F022h

STORE (Head)

INC Head

LOAD Head

CMP End

JNZ SKIP0

LOAD Start

STORE Head

SKIP0:		INC Cnt

INTE

INC Cnt

INTE

JMP LOOP0

INT_PER3:

PUSHA

LOAD (Tail)

OUT F032h

INC Tail

LOAD Tail

CMP End

JNZ SKIP3

LOAD Start

STORE Tail

SKIP3:		DEC Cnt

JNZ EXIT3

POPA

STORE Temp

POPA

AND #FFFEh

PUSHA

LOAD Temp

PUSHA

EXIT3:		POPA

RTI