Rešenja zadataka iz Arhitekture i organizacije računara 2, jun 2007.

1.

a)

b)		; Instruction fetch

BEGIN:	PCout,MARin,Xin

incA,ALUout,PCin

read

MDRout,IRin

; Instruction decode

opcase	; on an illegal opcode, go to INTH

; PUSH

PUSH:		PUSHadmod	; on an illegal addressing mode, go to INTH

; Register direct

PUSHRD:	REGout,MDRin

PUSHOP:	SPout,MARin,Xin

incA,ALUout,SPin

write

branch(,BEGIN)

; Immediate

PUSHIM:	PCout,MARin,Xin

incA,ALUout,PCin

read

branch(,PUSHOP)

; Register indirect:

PUSHRI:	PCout,MARin,Xin

incA,ALUout,PCin

read

MDRout,Yin

REGout,Xin

add,ALUout,MARin

read

branch(,PUSHOP)

; Interrupt handler for the internal interrupt

INTHIRQ:	SPout,MARin,Xin

PCout,MDRin

incA,ALUout,SPin

write

SPout,MARin,Xin

PSWout,MDRin

incA,ALUout,SPin

write

IVTEout,Xin

IVTPout,Yin

add,ALUout,MARin

read

MDRout,PCin,clIRR,clT,branch(,BEGIN)

c)		Postoje indikatori: JZ, znaèenje: pop addr, if ZF then goto addr

Ne postoje indikatori: JZ, znaèenje: pop addr, pop val, if val=0 then goto addr

Analogno za ostale indikatore, odnosno relacije.