1.	b)

; Instruction fetch

BEGIN:	PCout,MARin,Xin

read,incA,ALUout,PCin

wmfc

MDRout,IRin

; Instruction decode

opcase

; JMP instruction

JMP:		PCout,MARin,Xin

read,incA,ALUout,PCin

wmfc

MDRout,TEMPin,Xin

admodjmp	; JMP addr. mode

JMPABS:	TEMPout,PCin,branch(,END)

JMPREL:	PCout,Yin

add,ALUout,PCin,branch(,END)

JMPRIND:	regsel0,REGout,Yin

add,ALUout,PCin,branch(,END)

JMPMIND:	regsel0,REGout,Yin

add,ALUout,MARin

read

wmfc

MDRout,PCin,branch(,END)

END:		branch(IR,INTH)

branch(,BEGIN)