2.  a)

| Šta? | Početna adresa | Poslednja adresa |
| --- | --- | --- |
| Memorijski adresni prostor | 0000 0000h | BFFF FFFFh |
| Ulazno-izlazni adresni prostor | C000 0000h | FFFF FFFFh |
| Prostor rezervisan za ROM memoriju | A000 0000h | BFFF FFFFh |
| Fizička RAM memorija | 0000 0000h | 1FFF FFFFh |
| Fizička ROM memorija | B000 0000h | BFFF FFFFh |

b) xxxx=FF00h	c)

| AddressBus | DataBus | BusCycleType | Comments |
| --- | --- | --- | --- |
| 0000 0100 | 0000 | MRd | Instruction Fetch |
| 101 | 2140 | MRd |  |
| 102 | 0000 | MRd |  |
| 103 | 0002 | MRd |  |
| 2 | 0000 | MRd | Fetch Address (Op #2) |
| 3 | 010A | MRd |  |
| 10A | FF00 | MRd | Fetch (Op #1) |
| 10B | 0000 | MRd |  |
| 104 | 2300 | MRd | Instruction Fetch |
| 105 | FF00 | MRd |  |
| 1000 | 0000 | MWr | Save PC on stack |
| 1001 | 0106 | MWr |  |
| 200 | 0042 | MRd | Instruction Fetch |
| 201 | 0020 | MRd |  |
| 1002 | 0000 | MWr | Execute PUSH instruction |
| 1003 | 0200 | MWr |  |
| 202 | 0020 | MRd | Instruction Fetch |
| 203 | 0203 | MRd |  |
| 204 | 0000 | MRd |  |
| 205 | 0006 | MRd |  |
| 206 | 0AA0 | MRd | Instruction Fetch |
| 207 | 0000 | MRd |  |
| 1003 | 0200 | MRd | Get PC from stack |
| 1002 | 0000 | MRd |  |
| 200 | 0042 | MRd | Instruction Fetch |
| 201 | 0020 | MRd |  |
| 1002 | 0000 | MWr | Execute PUSH instruction |
| 1003 | 0206 | MWr |  |
| 202 | 0020 | MRd | Instruction Fetch |
| 203 | 0203 | MRd |  |
| 204 | 0000 | MRd |  |
| 205 | 0006 | MRd |  |
| 206 | 0AA0 | MRd | Instruction Fetch |
| 207 | 0000 | MRd |  |
| 1003 | 0206 | MRd | Get PC from stack |
| 1002 | 0000 | MRd |  |
| 206 | 0AA0 | MRd | Instruction Fetch |
| 207 | 0000 | MRd |  |
| 1001 | 0106 | MRd | Get PC from stack |
| 1000 | 0000 | MRd |  |
| 106 | 0032 | MRd | Instruction Fetch |
| 107 | 7051 | MRd |  |
| 108 | 0000 | MRd |  |
| 109 | 0000 | MRd |  |
| 6 | 4040 | MRd | Operand Fetch |
| 7 | 00FF | MRd |  |
| 0 | 4040 | MWr | Store Results |
| 1 | 00FF | MWr |  |
| 10A | FF00 | MRd | Instruction Fetch |
| 10B | 0000 | MRd |  |

4.	Glavni program:

; init input

CLRA

STORE Sin

LOAD #1000h

STORE AdrBin

STORE AdrBinSave

LOAD #200h

STORE CntBin

OUT #87h

STORE F010h	; St. Per0

;.....

WaitF:	LOAD Sin

AND #1

JZ WaitF

; =============;

ChDir:	LOAD AdrBinSave

OUT F003h	; src address

LOAD #1200h

STORE AdrBin

STORE AdrBinSave

LOAD #1

STORE Dir

InitA:	CLRA

STORE Sin

STORE Sm2m

LOAD #200h

STORE CntBin

STORE CntBout

OUT F005h

LOAD #1400h

OUT F004h	; dest address

STORE AdrBout

LOAD #85h	; start DMA

OUT F000h

LOAD #87h	; start Per0

OUT F010h

; ....

WaitM:	LOAD Smtom

AND #1

JZ WaitM

; =============;

LOAD #1

OUT F020h	; Start Per1

CheckRdy:	IN F021h

AND #8000h

JZ CheckRdy

LOAD (AdrBout)

SHL 1	; *2

OUT F022h

INC AdrBout

DEC CntBout

JNZ CheckRdy

CLRA

OUT FF20h

;.......

WaitS:	LOAD Sin

AND #1

JZ WaitS

LOAD Dir

AND #1

JZ ChDir

; change Dir

LOAD AdrBinSave

OUT F003h	; src address

LOAD #1000h

STORE AdrBin

STORE AdrBinSave

CLRA

STORE Dir

JMP InitA

Prekidna rutina za DMA:

IntDma:	CLRA

OUT F000h

INCA

STORE Smtom

RTI

Prekidna rutina za PER0:

IntPer2:	IN F012h

STORE (AdrBin)	; ;put a(i) into Buf

INC AdrBin

DEC CntBin

JNZ Back

; stop Per0

OUT F010h

INCA

STORE Sin

Back:	RTI

b) NE. Procesor ne može izvršiti ciklus čitanja na magistrali jer Dma kontroler radi u burst režimu i drži magistralu zauzetu do završetka prenosa celoh bloka podataka.