Rešenja zadataka iz Arhitekture računara, Septembar 1998.

1.	a)

b)					c)

3. a)	Procesor prihvata prekid istog nivoa kao što je tekući. U PSW postoji samo jedan bit L koji treba da koduje nivoe glavnog programa i prekida sa IRQ0 i IRQ1, što znači da glavni program i prekid nižeg nivoa (sa IRQ0) dele isti kod 0. Kako zato nema načina da se razlikuje da li je procesor trenutno u glavnom programu ili u prekidnoj rutini za IRQ0, mora prihvatiti novi prekid sa IRQ0 (jer nema smisla da ga odbije ukoliko je u glavnom programu, što bi značilo da se on tada nikako ne može obraditi), čak i ako se nalazi u prekidnoj rutini za prekid sa IRQ0.

b)	PER0 na IRQ0, PER1 na IRQ1. U toku prekidne rutine za PER0 stiže zahtev za prekid od PER1 i prihvata se, što znači da je višeg prioriteta, jer je vezana na različitu liniju.

c)	LOAD #0

STORE 08h ;16-bitna operacija

STORE 10h

LOAD #AA00h

STORE 0Ah

LOAD #BB00h

STORE 12h

d)	01

01

00

00

PSWL: L:I=01

PSWH

01

AA

00

00

PSWL: L:I=11

Vrh->	PSWH

e)	100, AA00, BB00, BB01, BB02, BB04, BB05, AA01, AA02, AA00, AA01, AA02, 101

- a)

- Memorijski adresni prostor (RAM + ROM):	4000 0000h - FFFF FFFFh

RAM: 4000 0000h – BFFF FFFFh

ROM: C000 0000h – FFFF FFFFh

UI adresni prostor:	0000 0000h – 3FFF FFFFh

c)

d)

4.	Glavni program:

; init input from PER0

LOAD #1

STORE SemIB	; first time

LOAD #1000h

STORE AdrUB

LOAD #200h

STORE CntUB

LOAD #1h

OUT F010h	; Start Per0

ChkRdy0:	IN F011h

AND 8000h

JZ ChkRdy0

IN F012h

STORE (AdrUB) ; put a(i) into Buf

INC AdrUB

DEC CntUB

JNZ ChkRdy0

OUT F010h	; Stop PER0

WaitRdyIB:	LOAD SemIB

AND #1

JZ WaitRdyIB

; DMA transfer: UB to IB

LOAD #1000h

OUT F003h

LOAD #1200h

OUT F004h

LOAD #200h

OUT F005h

CLRA

STORE SemMtoM

LOAD #89h

OUT F000h;	Start DMA

...

WaitMtoM:	LOAD SemMtoM

AND #1

JZ WaitMtoM

; start input + output

LOAD #1200h

STORE AdrIB

LOAD #1000h

STORE AdrUB

LOAD #200h

STORE CntUB

STORE CntIB

CLRA

STORE SemIB

LOAD #1

OUT F010h

LOAD #8Dh

OUT F020h

JMP ChkRdy0

Prekidna rutina za DMA:

IntDma:		CLRA

STORE F000h

INCA

STORE SemMtoM

RTI

Prekidna rutina za PER1:

IntPER1:	LOAD (AdrIB)

OUT F022h

INC AdrIB

DEC CntIB

JNZ Back

OUT F020h; Stop PER1

INCA

STORE F000h

INCA

STORE SemIB

Back:		RTI

- 804h puta

- 4 puta