Elektrotehnički fakultet u Beogradu	Jun 1998.

Katedra za računarsku tehniku i informatiku

13. Novembar 1999.

Pismeni ispit iz Arhitekture računara, Oktobar II 1999.

#### Rešenja zadataka

1.

3.	3.	a) Prva reč sadrži kôd operacije i način adresiranja, a druga 16-bitnu adresu MemCnt.

b) 4.		d) PER0 vezati na IRQ1, a PER1 na IRQ0.

c)	LOAD	#23F7h

STORE	7

e) Na početak prekidne rutine PER0 treba dodati:	PUSH	PSW	; Push PSW

POPA		; Pop ACCumulator

OR	#11b	; ACC[1]=1,ACC[0]=1

PUSHA	; Push ACC

POP	PSW	; Pop PSW (L=11).

- a)

b)

c) trivijalno

4.

Glavni program:

; initialize input from PER0 (fill BP0)

LOAD #0

STORE Mfin

LOAD #1000h

SOTRE Ain

LOAD #100h

STORE Cntin

LOAD #5

OUT FF10h

; ...

Wait1:	LOAD Mfin

AND #1

JZ Wait1

; initialize output from BP0 and input to BP1

Swap:		LOAD #1000h

STORE Aout

LOAD #1100h

STORE Ain

LOAD #1

STORE Dir

Init:		LOAD #0

STORE Mfin

STORE MFout1

LOAD #100

STORE Cntin

; init DMAs

LOAD Aout

OUT FF01h

STORE Adrout

LOAD #100h

OUT FF03h

STORE Cntout

; start controllers

LOAD #5

OUT FF10h

LOAD #Fh

OUT FF00h

LOAD #3h

OUT FF20h

LOAD #3h

OUT FF30h

Check2:	IN F031h

AND #10h

JZ Check2

LOAD (Adrout)

OUT F032h

INC Adrout

DEC Cntout

JNZ Check2

OUT F300

;		....

Wait2:	LOAD Mfin

AND MFout1

JZ Wait2

LOAD Dir

AND #1

JZ Swap

; initialize output from BP1 and input to BP0

LOAD #1100

STORE Aout

LOAD #1000h

STORE Ain

LOAD #0

STORE Dir

JMP Init

Prekidna rutine:

DMA1Int:	LOAD #0

STORE FF20h

STORE FF00h

INC

STORE MFout1

RTI

Per0Int:	LOAD FF12h

STORE (Ain)

INC Ain

DEC Cntin

JNZ Back0

LOAD #0

OUT FF10h	;stop PER0

INC

STORE MFin

Back0:	RTI

b) 4112