1.		BEGIN:	PCout,MARin,incB,ldY

read,Yout,PCin

wmfc

MDRout,IRin

Instruction decode	opcase	; on an illegal opcode, go to INTH

ALU binary instructions	ALUBIN:	ALUadmod

Immediate	ALUIMM:	PCout,MARin,incB,ldY ; Fetch immediate operand

read,Yout,PCin

wmfc

ACCout,Xin

MDRout,ALUop,ldPSW,ldY,branch(CMP,ALUEND)

Yout,ACCin,branch(,ALUEND)

Memory direct LUMD:	PCout,MARin,incB,ldY ; Fetch operand address

read,Yout,PCin

wmfc

MDRout,MARin

read,ACCout,Xin

wmfc

MDRout,ALUop,ldPSW,ldY,branch(CMP,ALUEND)

Yout,ACCin,branch(,ALUEND)

; Based-Indexed

ALUBI:	regsel1,REGout,Xin

regsel2,REGout,ALUadd,ldY

Yout,MARin

read,ACCout,Xin

wmfc

MDRout,ALUop,ldPSW,ldY,branch(CMP,ALUEND)

Yout,ACCin,branch(,ALUEND)

ALUEND:	branch(IRQ,INTH)

branch(,BEGIN)

c)		LOAD	#0	; Sum:=0

STORE	Sum

LOAD	#a	; R0:=a, base (row) address

STORE	R0

LOAD	#10	; R2:=10, row counter

STORE	R2

ELOOP:	LOAD	#0	; R1:=0, index (element pointer)

STORE	R1

LOAD	#20	; R3:=20, element counter

STORE	R3

ILOOP:	LOAD	Sum

ADD	[R0][R1]

STORE Sum

LOAD	R1	; Next element

INC

STORE	R1

LOAD	R3

DEC

STORE	R3

JNZ	ILOOP

LOAD	R0	; Next even row

ADD	#40

STORE	R0

LOAD	R2

SUB	#2

STORE	R2

JNZ	ELOOP