Rešenja Arhitekture i organizacije računara 2, Septembar 2006.

1.	a); Instruction fetch

BEGIN:	PCout,MARin,Xin

read,incA,ALUout,PCin

wmfc

MDRout,IRin

; Instruction decode

opcase

; MOV instruction

MOV:		admodMOV	; switch on address mode

; Immediate

MOVIMM:	PCout,MARin,Xin

read,incA,ALUout,PCin

wmfc

MDRout,regsel1,REGin	; store to destination

; Register direct

MOVRD:	branch(DIR,MOVRD2)	; branch on direction

regsel2,REGout,TEMPin	; REG1 is destination

regsel1,REGin,TEMPout,branch(,END)

MOVRD2:	regsel1,REGout,TEMPin	; REG1 is source

regsel2,REGin,TEMPout,branch(,END)

; Register indirect with displacement

MOVRI:	PCout,MARin,Xin		; fetch displacement

read,incA,ALUout,PCin

wmfc

MDRout,Yin			; calculate address

regsel2,REGout,Xin

ALUadd,ALUout,MARin,branch(DIR,MOVLOAD) ; branch on direction

MOVSTORE:	regsel1,REGout,MDRin	; REG1 is source

write				; store to memory

wmfc

branch(,END)

MOVLOAD:	read				; REG1 is destination

wmfc

regsel1,REGin,MDRout,branch(,END) ; store to REG1

; Memory direct

MOVMD:	PCout,MARin,Xin		; fetch address

read,incA,ALUout,PCin

wmfc

MDRout,MARin,branch(DIR,MOVLOAD) ; prepare address

branch(,MOVSTORE)

END:		branch(IRQ,INTH)		; Branch on interrupt

branch(,BEGIN)

b)			MOV	R1,head		; for (cur=head;

LOOP:		CMP	R1,#0			; cur!=0 ?

JZ	EXIT

CMP	e[R1],R0		; if (cur->e == x)

JZ	EXIT			;    break;

MOV	R1,next[R1]		; cur = cur->next

JMP	LOOP

EXIT:

c)	Instrukcija:	JZ	Ri, addr

Značenje:	if (Ri==0) goto addr